EE8218 Final Exam , Dec 11, 2017            Time 1:30

Name——————————————————

Q1-( 10 marks) What are the advantages and limitations of shared memory multiprocessor system

Q2- (10 marks)  What are the advantages and limitations of distributed memory multiprocessor system with message passing

Q3-(10 marks) What are the advantages  and limitations of GPU based system

Q4-(10 Marks)

A-List the different steps required to write an efficient parallel code

Q4-B-In what step the parallel code must use the specific parallel computer model and explain it by giving example for shared memory versus message passing

Q5-(60 marks)

Compare the scalability of shared memory, distributed memory system and GPU based to a single processor by constructing a simple analytical models and assume the following:

-Application is equation solver 2000X2000 elements and each element is 32 bits and number of processors 100.

-Each computation per element is 30 instructions, and each instruction average time is 1 ns.

-Computation takes 50 times to converge

-Cost of synchronization 100 instructions per shared element

-Cost of communication= 20 us latency and Bandwidth is 200 Mbps assuming 10% of communication cost could be overlapped with computation.

-GPU cost of transfer data 2000x2000 elements over PCI BUS of BW= 10 Gbps to GPU and the results back to host