Digital  Systems  Engineering COE758

Final Exam, Date:  Dec 3, 2013

Time: 2 hours and 30  Minutes

Answer all the following questions

Calculators Allowed

Student Name ————————————————————

----------------------------------------------------------------------------------------------------------

Q1 – =10 marks

Explain the reasons for the following:-

A-Set Associativity Improves cache Hit rate

B-Using LRU Improves Cache Hit rate

C-Using separate data and instruction cache has worse hit rate compared to unified cache, but gives better performance

1

D-Write Allocate gives better performance than No Write Allocate

Q2- =12 marks

A-Calculate the total size of a 128 KB, two way set associative cache, if block size = 16 B, uses 1 dirty bit and assume processor address= 32 bit.

Q2-B- In the above cache, calculate the tag of an access to address 005E7000 HEX.

2

Q2-C- Draw the block diagram of the above cache showing tags, data cache and

buses connecting processor data and address to cache.

Q3- =12 Marks

A-Calculate the size of page table for 1 MB physical memory if page size= 16 KB, and page table uses 1 extra bit for valid. Assume processor Virtual address = 32 bits.

3

B- Calculate the size of TLB,  if the above system uses two way set associative TLB with 32 enteries for each set and one bit for valid.

C-In the above system, translate the virtual address 0A00E308  to physical address, assuming this  Virtual page maps to physical page 10.

4

Q4- = 10 Marks

A-Draw the timing diagram of Synchronous bus Read assuming device latency = 2 cycles, one cycle to send address and one cycle to send data.

B-Explain by giving a block diagram the operation of a Central Arbiter and how a grant is generated for each device.

5

C- What are the parameters that affect the transfer time of a block from hard disk

Q5=10 Marks

Q5-A- Explain how and why a DMA is used to transfer data from a device

Q5-B-Why DMA use might cause problem to cache system

6

Q5-C-Assume a 1 GHz processor that uses DMA to transfer 16 KB from H.D to

Memory. The setup time of DMA transfer = 1000 cycles and the interrupt requires 400 cycles. The transfer rate is 2 MB/sec.

Calculate the fraction of processor time consumed in the transfer

Q6=12 Marks

Q6-A-List the main components used in distributed shared memory multiprocessor system and draw a block diagram.

Q6-B- Assume a shared memory distributed multiprocessor system and X1 in P1 memory, and X2 is in P2 memory. Give the directory state , bus operation and cache state for the following:-

-P2 reads X1

-P1 reads X2

-P1 Writes to X1

-P2 reads X1

7

8